# Road Crash Hotspot Analysis — Birmingham, 2020–2024

### Findings & Recommendations Report

## Executive Summary

This analysis examined 10,817 recorded road traffic collisions in Birmingham, England, between 2020 and 2024, using DfT STATS19 road safety data. The objective was to identify statistically significant crash hotspots — locations where collision frequency is too high to be explained by chance — and to provide the city's road safety stakeholders with an evidence-based basis for prioritizing interventions.

Spatial statistical testing confirmed that crashes in Birmingham are not randomly distributed: they cluster strongly and significantly (Moran's I = 0.56, p = 0.001). Using Getis-Ord Gi* hotspot analysis on a 500m grid, 105 grid cells (5.5% of the study area) were identified as statistically significant hotspots, with 36 cells reaching the highest confidence tier (99%). These hotspots concentrate heavily around Birmingham's city centre and along a corridor extending south/southwest toward Edgbaston, suggesting that a small number of locations account for a disproportionate share of the city's collision burden.

## 1. Background & Objective

Problem statement: Birmingham's road safety team (or equivalent local authority stakeholder) needs to know where to prioritize limited road safety intervention resources — safety audits, signage, lighting improvements, speed enforcement — rather than spreading efforts evenly across the city.

Study window: 2020–2024 (5-year window; 2025 excluded as only provisional STATS19 data is currently available, with final data due July 2026).

Study area: Birmingham Local Authority District (ONS code E08000025).

## 2. Data & Methodology

Data source: DfT STATS19 Road Safety Data — Collision-level records, filtered to Birmingham LAD, 2020–2024.

Tools used: Python (pandas, geopandas, PySAL/esda) for data processing and spatial statistics; SQL (SQLite) for structured querying; QGIS for cartographic visualization; [Power BI — pending] for the interactive dashboard.

Methodology steps:

- Filtered STATS19 collision data to Birmingham (ONS code E08000025), 2020–2024 → 10,817 records

- Geocoded to point locations, reprojected to British National Grid (EPSG:27700) for accurate distance-based analysis

- Aggregated crash points into a 500m × 500m grid (1,911 cells total) — standard practice for point-pattern hotspot analysis, avoiding the instability of running statistics directly on raw point locations

- Tested for global spatial autocorrelation using Moran's I, to confirm clustering exists before proceeding

- Ran Getis-Ord Gi* local statistics to identify which specific cells are statistically significant hot (or cold) spots, classified into 90%, 95%, and 99% confidence tiers

- Cross-referenced the highest-confidence hotspot cells against real-world locations (via coordinate lookup) to ground the statistical findings in actual geography

## 3. Key Findings

3.1 Crashes are strongly and significantly clustered, not randomly distributed. Moran's I = 0.56 (p = 0.001, z = 49.6) — a result well beyond the threshold for statistical significance, confirming that hotspot analysis is a valid and meaningful approach for this dataset.

3.2 A concentrated core of high-confidence hotspots exists in and around Birmingham city centre. The two highest-intensity cells (z = 5.83 and z = 5.54, both significant at 99% confidence) sit near the city centre/Digbeth-Southside area, with 103 and 98 recorded crashes respectively within their 500m cells over the 5-year window — far above the study area's cell average of 5.7 crashes.

3.3 Hotspots extend along a corridor pattern, not just isolated points. Beyond the central core, a secondary cluster of 95–99% confidence cells traces a corridor toward Edgbaston/Ladywood, consistent with a busy arterial route (likely A38/A4540 ring road vicinity) rather than a single junction — suggesting the underlying risk factor may be corridor-wide traffic volume/geometry rather than one specific black-spot.

3.4 Total crash volume dipped in 2020, then rose through 2024. Annual totals: 2020 (1,802) → 2021 (2,137) → 2022 (2,383) → 2023 (2,199) → 2024 (2,296). The 2020 dip is consistent with reduced traffic volumes during COVID-19 restrictions; the subsequent rise likely reflects a return to pre-pandemic traffic levels.

3.5 Serious-injury crashes have trended upward even as overall patterns fluctuated. Serious-severity crashes rose from 308 (2020) to 459 (2024), a steeper increase than the overall volume trend — worth flagging as a distinct concern separate from raw crash counts.

[Placeholder — fill in once you've manually reviewed the top hotspot locations via satellite/street view]: 3.6 Contributing factors at top hotspot locations: [e.g., "The two highest-confidence cells both correspond to signalized junctions on major arterial roads with high pedestrian footfall" — replace with your actual observations]

## 4. Recommendations

(Draft — refine once contributing-factor analysis from Section 3.6 is complete)

- Prioritize a detailed engineering safety audit of the top 5 highest-confidence (99%) hotspot cells, starting with the two highest-intensity locations near the city centre.

- Treat the identified corridor cluster as a single intervention zone rather than isolated points — corridor-wide measures (speed management, junction phasing review) may be more effective than point treatments alone.

- Investigate the rising serious-injury trend independently from overall crash volume — this may reflect changes in vehicle mix, speed, or road user behavior that warrant separate analysis.

- Cross-reference hotspot locations against planned infrastructure works — if any hotspot cells overlap with roads already scheduled for resurfacing or redesign, safety improvements could be bundled into existing capital projects at lower marginal cost.

## 5. Limitations

- Grid-based aggregation (500m cells) necessarily smooths some spatial precision — a crash at a specific junction and a crash 400m down the same road may fall in the same cell.

- STATS19 only records reported collisions to police; near-misses and unreported minor incidents are not captured.

- This analysis is descriptive/diagnostic (identifying where), not causal (fully explaining why) — Section 3.6 findings are observational and would benefit from traffic engineering site review before being treated as conclusive.

- 2025 data was unavailable as final/validated STATS19 release at time of analysis (provisional data expected to finalize July 2026).

## 6. Next Steps for This Project

- [ ] Complete Section 3.6 with actual site observations from top hotspot locations

- [ ] Build interactive Power BI dashboard for stakeholder exploration (filters: year, severity, time of day, road type)

- [ ] Cross-tabulate hotspot cells against weather/lighting conditions to strengthen contributing-factor analysis

- [ ] Finalize QGIS map export for portfolio/report inclusion

- [ ] Publish summary write-up (LinkedIn/portfolio site) condensing this report into a shorter public-facing version